Date:13/10/26

ITA-0329:MOBILE COMPUTING

CO1-AT2-Short Answer Test

# Q1.)

Given,

Total spectrum=45 MHz

Bandwidth per simplex channel=25 kHz.

A full-duplex channel requires two simplex channels.

= 45,000 kHz/(2*25 kHz)

= 45,000/50=900

# Q2.)

Given,

Total voice channels=560 frequency reuse cluster size=7 cells.

Channels per cell=Total channels/Cluster size

= 560/7=80

Name:S.Dinesh Regiser No:192311302

# Q3.)

Given,

Number of carriers=100 Time slots per carrier=8.

Total simultaneous voice channels=Number of carriers*Time slots per carrier

= 100*8=800

# Q4.)

Given,

Base-station transmit power=80 W Handset transmit power=4 W Power ratio(dB)=10 log₁₀(P₁/P₂)

=10 log₁₀(80/4)

=10 log₁₀(20) ≈ 13.01 dB

# Q5.)

Given,

Carrier power=−60 dBmI Interference power=−92 dBm.

C/I (dB)=Carrier power(dBm)−Interference power(dBm)

=−60−(−92)=32dB

# Q6.)

Given,

Total spectrum=42 MHz Channel bandwidth=300 kHz Reuse cluster size=7 cells.

Total channels=42,000kHz/300kHz=140 Channels per cell=140/7=20

# Q7.)

Given,

Initial received power=64 W Path-loss exponent n=4.

P₂ = P₁(d₁/d₂)^n

= 64×(1/2)^4

= 64×1/16=4W

# Q8.)

Given,

Path-loss exponent n=2

Required power scaling factor=(R₂/R₁)^n

=(2R₁/R₁)^2=2^2 =4

# Q9.)

Given,

Cluster size=4 cells

Each cell is provisioned with 15 voice channels Total channels in the cluster=4*15=60

# Q10.)

Given,

Vehicle speed=100 km/h Carrier frequency=900 MHz Speed of light (C)≈3*10⁸ m/s.

Convert speed(V)=100/3.6≈ 27.78 m/s

Maximum Doppler shift: fD = Vf/C

=(27.78*900*10^⁶)/(3*10^⁸)≈83.33 Hz

# Q11.)

Given,

TDMA frame duration=5 ms Number of equal slots=8.

Slot duration=Frame duration/Number of slots

=5ms/8=0.625 ms

# Q12.)

Given,

Allocated bandwidth=24 MHZ Carrier spacing=200 KHZ

Number of carriers=24,000KHZ/200KHZ=120

# Q13.)

Given,

Transmit power=8W Path loss=33dB

Convert transmit power to dBm:Pₜ=10 log₁₀(8,000mW)≈39.03dBm Received power: Pᵣ=Pₜ−path loss

=39.03−33=6.03dBm

# Q14.)

Given, Frequency=1900MHZ Distance=2.5 km

FSPL(dB)=32.44+20log₁₀(f in MHz)+20log₁₀(d in km)

=32.44+20log₁₀(1900)+20log₁₀(2.5)

≈ 105.97dB

# Q15.)

Given,

Number of cells=6

Each cell supports 250 simultaneous calls Cluster capacity=Number of cells*Calls per cell

= 6*250=1,500

# Q16.)

Given,

Call arrival rate=4,000 calls per second Average call duration=3 minutes=180 seconds

Number of active calls = Arrival rate*Average duration

=4,000*180=720,000

# Q17.)

Given,

Number of channels=150 Bandwidth per channel=25kHz.

Total bandwidth=150*25kHz=3,750kHz=3.75MHz

# Q18.)

Given,

30 adjacent frequency bands Each gap is 40 kHz

There are 29 gaps between 30 bands

Total guard-band bandwidth =(30−1)*40kHz

=29*40=1,160 kHz=1.16MHz

# Q19.)

Given,

Vehicle speed = 90 km/h cell radius = 1.5 km

Cell diameter = 2*1.5 = 3 km

Approximate handoffs per hour = 90 km/h/3 km = 30

# Q20.)

Given,

8 regional center

40 base stations per center 250 users per base station

Total capacity = 8* 40* 250 = 80,000

# Q21.)

Given,

Coverage area = 400 km² cell radius = 400 m = 0.4 km

Approximate area per cell = R^² = 0.4^² = 0.16 km^² Number of cells = 400 / 0.16 = 2,500

# Q22.)

Given,

Total spectrum = 6 MHz bandwidth per channel = 30 kHz

Convert 6 MHz to 6,000 kHz.

Number of channels = 6,000 / 30 = 200

# Q23.)

Given,

1,200 subscribers

Each makes 4 calls per hour Average call duration = 2.5 minutes

Traffic per subscriber = (Calls per hour*Duration in minutes) / 60

= (4*2.5) / 60 = 0.1667 Erlangs

Total traffic = 1,200* 0.1667 ≈ 200 Erlangs

# Q24.)

Given,

Total bandwidth = 40 MHz

Bandwidth allocated per user = 400 kHz. Maximum users = 40,000 / 400 = 100

# Q25.)

Given,

1,200 base stations

Maximum capacity per base station = 600 simultaneous users Total user capacity = 1,200* 600 = 720,000